Sensing Reality!

OLamView

Quick User Manual

Applicable to the following series of products

LR-1BSx，LR-1F,LR-16F...

Please read this manual before using the product for best product performance.

Be sure to keep this manual properly for future reference.

Doc#:OLA-OM-2.1CN

Contents

1.  oftware Installation	3

2.    Network Configuration	4

3.    Software interface	4

3.1 Use of menu bar	4

3.1.1 File menu	4

3.1.2 Tools menu	5

3.1.3 Views menu	7

3.2 Tools bar	8

3.2.1 Base tools bar	8

3.2.2 Color tools bar	8

3.2.3 Play tools bar	11

3.2.4 Views tool bar	11

3.2.5 Camera control tools	12

- Software Installation

Software runtime environment:

- Windows 7, Windows 8 or Windows 10 system

- No less than 2G memory capacity

- At least 100M free hard disk space

points  for attention：

- The Windows OS firewall must authorize the UDP communication of OlamView

- Ensure that the installation path is in English characters, A void software adaptation failures caused by non ASCII characters

- Open ” OlamView install.exe”

Figure 1 Software Icon

- Network Configuration

- The communication between LiDAR and computer adopts Ethernet medium; Using UDP protocol; The factory default is static IP and port.

Basic communication parameters:

| LiDAR IP | 192.168.1.100(default) | Allow editing |
| --- | --- | --- |
| Host IP | 192.168.1.10(default) | Allow editing |
| Host Port | 2368(default) | Allow editing |
| Netmask | 255.255.255.0 | Allow editing |
| Gateway | 192.168.1.1 | Allow editing |

figure 2 network

- Software interface

3.1 Use of menu bar

3.1.1 ‘File’ menu

- Click ‘Open’ and select ’Capture File’ menu, Open the original LiDAR communication data document in PCAP format. This function is used to browse the original LiDAR communication data document saved in PCAP format.

- Click ‘open’ and select ‘sensor stream’ to connect to the online LiDAR of the local computer, After checking ‘advanced configuration’, you can set LiDAR port, local IP and other parameters. The default value is the factory default parameter of LiDAR.

- Click  ‘Recent Files’ menu, Open recent files

- Click ‘Save As’ menu, Saved the received LiDAR point cloud information into pcap and other formats.

- Click ‘Save  Screenshot’ menu, Saved screenshot of current interface to local disk.

- Click ‘Close Data’ menu, Disconnect from the current LiDAR and clear the current content.

- Click ‘Exit’ menu, Closed Software.

3.1.2 Tools menu bar

- Click spreadsheet to open the LiDAR data table and display the detailed data of radar point cloud in spreadsheet.

Displays the data of the selected points cloud，Displays the data of all points

- Click ‘measurement grid’ to turn the grid on or off。

- Click ‘Grid Properties’：

Sensor origin, Changed LiDAR origin

Grid resolution, Set grid resolution

Grid radius, Set grid radius

Grid line width, Changed grid line width

Grid color, Set grid color

5. Click ‘Sensor Configuration’, Set parameters related to LiDar sensor

6. Click ‘Laser Selection’, Select channel， Display more corrections, Vertical offset, horizontal offset and other values can be displayed

7.  Click ‘Crop Returns’, Enable Cropping of returns.

8. Click   , Play the point cloud data that has been loaded into olamview.

9. Click  , Record the radar point cloud data being connected in real time into radar point cloud document in PCAP format

3.1.3 Views menu bar

3.2 Tools menu bar

3.2.1 Base menu bar

1. Click , Open the points cloud data

2. Click , Connect to the online LiDAR of the local computer

3. Click , LiDAR conifg

4. Click , Point cloud clipping

5. Click , Circle a rectangular area and select all points in this area

6. Click , Open the LiDAR data table and display the detailed data of radar point cloud in spreadsheet

7. Click , Open python console

8. Click , Open error console

3.2.2 Color tools bar

- Click  open the color view，

Color rendering is the visualization of point cloud data and provides comparison of data differences.

You can visually render data in different dimensions through the options in the drop-down list There are options such as distance, angle, signal strength of target object and so on.

- Click   Click map editor

Click  open setting:

- Lock Data Range: When checked, lock the color board so that it will not automatically adapt to the current data range

- Interpret Values As Categories

- Rescale On Visibility Change

- Click  Reset the data range of the color board

- Click  Change the data range of the ruler

- Click  Change ruler range based on Timestamp

- Click  Change ruler range based on visibility

- Click  Flip color settings

- Click  Select the preset color scale color scheme

- Click  Save as preset scheme

- Mapping Data：Adjust color palette

- Use log scale when mapping data:

- Color Space：

- Discretize：

- Use Below Range Color:

- Use Above Range Color:

Color Legend

- Auto Orient：Color palette orientation adaptation

- Orientation： select the direction of the color board

- Title：Sets the title of the color palette

- Component Title：sets the Component title

- Title Justification：Title Justification

- Title Font Properties：Title Font Properties

- Text/Annotation Font properties：Text/Annotation Font properties

- Automatic Label Format：Automatic Label Format

- Label Format ：Label Format

- Maximum Number of：Maximum Number

- Draw Tick Mask：Draw Tick Mask

- Draw Sub Tick Marks：Draw Sub Tick Marks

- Draw Tick Labels：Draw Tick Labels

- Add Range Labels

- Range Labels Format

- Draw Annotations

- Add Range Annotations

- Automatic  Annotations

- Draw Nan Annotation

- Nan Annotation

- Tick and Annotation Positions

- Aspect Ratio

- Click  reset

3. Click  Adaptively reset the data range of the color board

4. Click   Change the data range of the color version

5 From the drop-down menu, you can select the rules of color scale contrast

.

3.2.3 Play tools bar

1. Click  , Loop Playback

2. Click , Save captured data when connecting LiDAR

3. Click speed, Changed speed of play

4.  TF, time of frames

3.2.4 View tools bar

- , Switch to orthographic projection view

- , Open the measurement, press and hold control and the left mouse button to select two points to measure the distance between them (you need to open the orthogonal projection view first)

3.2.5 Camera angle control tool

- The camera viewing angle can be magnified back to the initial value

- Zoom to data area

- Enlarge the area drawn by the left mouse button

- The field of view will turn to the positive direction of the X axis

- The field of view will turn to the negative direction of the X axis

- The field of view will turn to the positive direction of the Y axis

- The field of view will turn to the negative direction of the Y axis

- The field of view will turn to the positive direction of the Z axis

- The field of view will turn to the negative direction of the Z axis

- Rotate 90 ° in positive direction

- Rotate 90 ° in negative direction

wechat

Specifications are subject to change without notice!

|  | Hangzhou ole-systems Co., Ltd. No.35 Jiuhuan Road, Jianggan District,Hangzhou, Zhejiang, China Web: www.ole-systems.com Email: sales@ole-systems.com Tel: 400 102 5850 |
| --- | --- |

© Copyright:Hangzhou ole-systems Co., Ltd.

release: 2020-12